# Scripts Tab

Linkcomm – Sutron Xlink500

# TASKS

## S1: alarm_in_fast

Settings:

Label: alarm_in

Script function: turn_on_sampling

Trigger: Alarm In Only

Functions:

- When an alarm is triggered (like M1 Alarm 1 exceeds threshold) the measurement intervals for level, flow, etc. are changed from 5min to 1min.

- Turns “sampling_on” to 1 (1=True, ON)

- Resets all event sampling parameters for the start of the event: volume_total to 0, bottle_num to 1, aliquots=0

## S2: alarm_out_slow

Settings:

Label: alarm_out

Script function: turn_off_sampling

Trigger: Alarm Out Only

Functions:

- When an alarm condition goes to OFF (In Measurements-Alarms, related to Alarm tx Mode) (like M1 Alarm 1 drops below threshold/deadband) the measurement intervals for level, flow, etc. are changed from 1min to 5min.

- Turns “sampling_on” to 0 (0=False, OFF)

## S3: turn_on_sampling

Settings:

Label: sampling_on

Script function: turn_on_sampling

Trigger: Digital 1 (dummy trigger, this doesn’t do anything, just has to have something here)

Functions:

- This must be manually triggered by selecting it in the Scripts window and clicking “Run Script Now”

- Turns “sampling_on” to 1 (1=True, ON)

- Resets all event sampling parameters for the start of the event: volume_total to 0, bottle_num to 1, aliquots=0

## S4: turn_off_sampling

Settings:

Label: sampling_off

Script function: turn_off_sampling

Trigger: Digital 1 (dummy trigger, this doesn’t do anything, just has to have something here)

Functions:

- This must be manually triggered by selecting it in the Scripts window and clicking “Run Script Now”

- Turns “sampling_on” to 0 (0=False, OFF)

## S5: reset_sampling_params

Settings:

Label: reset_sampl

Script function: reset_sampling_params

Trigger: Digital 1 (dummy trigger, this doesn’t do anything, just has to have something here)

Functions:

- This must be manually triggered by selecting it in the Scripts window and clicking “Run Script Now”

- Turns “sampling_on” to 1 (1=True, ON)

- Resets all event sampling parameters: volume_total to 0, bottle_num to 1, aliquots=0

# MEASUREMENTS

## Function Name: rating_table(stage_in)

### Usage: M2

#### Description:

Given stage reading, this script will find the closest stage/discharge pair in

rating table that is less than the input stage reading, and then perform a linear

interpolation on the discharge values on either side of the stage reading to

determine the discharge value at the current stage. For example, a stage value

of 4" would output 32.0 CFS discharge because 4 is between (3, 22) and (5, 42).

User will need to define the values for the rating table based on their application.

The example below assumes an input stage value in inches and outputs discharge in cubic feet

per second (CFS).

To configure this script, attach this function to a Stage measurement

or second meta referring to stage and make sure your stage units match your rating

table stage values.

:param stage_in: measured water level in inches (float)

:return: calculated flow from the HvF table (float)

## Function Name: compute_volume_total(flow_cfs)

### Usage: M7

#### Description:

This function needs to be associated with the total volume measurement (M7).

It will compute the total volume based on the current flow rate (from any flow data source) and past volume.

The script will trigger the sampler if appropriate.

:param flow: current flow rate

:return: the current volume reading

Dependencies: